Homework 2 answers

Note: These answers do not strictly display significant figures.

  - Using the rainfall summary for Washington, DC, for September 1983 (Table 4.1), graph the hourly hyetograph (in./hr.) for the storm of September 30.  Determine the total depth (in.).  Also, determine the peak hourly rate (in./hr.) and the maximum 3-hr rate of rainfall for the storm.  What volume of rain (in acre-ft) would fall on a 50-acre watershed for this storm event?

See Excel file.

  - Compute the average rainfall intensity (in./hr) for both of the following storms that occur on a 0.5-mi2 watershed: (a) A depth of 1.5 in. in 30 min and (b) a volume of 45 acre-ft during 40 min.

Intensity = Depth divided by Time (with Depth = Volume divided by Area):

(a)1.5in30min∗60minh=3inh(b)45ac⋅ft0.5mi2∗40min∗mi2640ac∗60minh∗12inft=2.53inh

  - Compute the total volume of rainfall (acre-ft) for eac of the following storms and watersheds: (a) Depth of rainfall = 3 in., drainage area = 0.25 mi2; (b) rainfall intensity = 0.6 in./hr, storm duration = 90 min, drainage area = 10 acres; and (c) depth of rainfall = 0.9 in., storm duration = 45 min, drainage area = 1 mi2.

(a)3in∗0.25mi2∗1ft12in∗640acmi2=40ac⋅ft(b)0.6inh∗90min*10ac∗1ft12in∗h60min=0.75ac⋅ft(c)0.9in∗1mi2∗1ft12in∗640acmi2=48ac⋅ft

  - Use Google to find 24-hour rainfall recurrence interval maps for Connecticut.  Find the 10 year, 24-hr depth (in.) and intensity (in./hr.) for Norwich, New Haven, and Canaan (up in the northwest corner of the Connecticut).  If you need a map to find those cities, try http://www.infoplease.com/atlas/state/connecticut.html.

Using “24-hour rainfall recurrence interval maps for Connecticut” produces http://pubs.usgs.gov/fs/fs-014-01/ as the first hit.  Figure 2 in that document is a map that gives the 10 year, 24 hour depths: Norwich, 4.4 inches or 0.18 in./hr; New Haven, 4.8 inches or 0.20 in./hr; Canaan, 5.0inches or 0.21 in./hr.

  - From the IDF curve shown in Figure 4.4, determine the missing values of intensity, depth, duration, or frequency for the following table:

| Intensity (in./hr) | Depth (in.) | Duration (hr) | Frequency (yr) |
| --- | --- | --- | --- |
| 2.2 | 1.65 | 0.75 | 5 |
| 0.7 | 4.2 | 6.00 | 25 |
| 8 | 2 | 0.25 | 100 |
| 5.0 | 2.5 | 0.50 | 50 |
| 0.3 | 7.2 | 24.00 | 100 |
| 4.0 | 2.7 | 0.67 | 50 |

  - Determine the average depth of rainfall for a 10-yr, 6-hr storm over a 150-mi2 in Baltimore (Figure 4.4).  Also, compute the total storm volume in acre-ft.

From the IDF curve, the intensity for a 10-yr, 6-hr storm is 0.59 in./hr.

Volume = Depth times Area (with Depth = Rate times Time):

0.59inhr∗6hr=3.54in0.59inhr∗6hr∗150mi2∗1ft12in∗640acmi2=2.8⋅104ac⋅ft